Harold Vance Department of Petroleum Engineering

Texas A&M University

## Petroleum Engineering 201S

Introduction to Petroleum Engineering

## Notes on Dimensions and Units

## Spring 2002

Fluid Properties

1.	Liquid Specific Gravity

γliquid = ρliquidρwater

2.	API Gravity

°API = 141.5γliquid − 131.5

3.	Gas Specific Gravity

γgas  = ρgasρair = MgMair = Mg(pure gas)28.97 = Mapparent( gas mixture)28.97

- Gas in Solution

# Rs =  standard cubic feet of gas liberated when one stock tank barrel of crude oil is

# produced

5.	Oil Formation Volume Factor

B = volume occupied at reservoir pressure and temperaturevolume occupied at standard pressure and temperature

B= .  In other words:

B= RBSTB

= mass of oilSTB+mass of gasSTBmass of reservoir liquidRB= ρSTO+0.01357RsγgρoR,

where:  and are densities of the stock tank oil and reservoir liquid, both in

lbm/ft3,  and 0.01357 converts the gas volume to mass.

The conversion factors is derived as follows:

RsScfSTBlb⋅mole380.7 Scf28.97γglbmlb⋅moleSTB5.615ft3= 0.01357Rglbmft3.

Example Fluid Properties Calculations

1.	Liquid Specific Gravity

Given

Density of Liquid		48.6 lbm/ft3

Density of Water		62.4 lbm/ft3

Definition

γliquid = ρliquidρwater

Calculation

γliquid = ρliquidρwater=48.662.4=0.779

2.	API Gravity

Given

Specific gravity of liquid-1		0.779

Specific gravity of liquid-2		0.876

Specific gravity of liquid-3		1.000

Definition

°API = 141.5γliquid − 131.5

Calculations

Specific Gravity	API Gravity

0.779		     50.0

0.876	                 30.0

1.000		     10.0

3.	Gas Gravity

Given		Component		Composition (Mole fraction)

Methane C1			0.850

Ethane C2			0.090

Propane C3			0.040

n-Butane n-C4			0.020

Definition

γgas  = ρgasρair = MgMair = Mg(pure gas)28.97 = Mapparent( gas mixture)28.97

3.	Gas Gravity (continued)

Calculation

| Component | Mole fraction | Molar Mass | Component Mass | Mass fraction |
| --- | --- | --- | --- | --- |
| j | yj | Mj | yjMj | yjMj19.26 |
| C1 | 0.850 | 16.04 | 13.63 | 0.708 |
| C2 | 0.090 | 30.07 | 2.71 | 0.141 |
| C3 | 0.040 | 44.10 | 1.76 | 0.091 |
| n-C4 | 0.020 | 58.12 | 1.16 | 0.060 |
|  | 1.000 |  | 19.26 | 1.000 |

γgas  = Mapparent( gas mixture)28.97 19.2628.97=0.665

4.	Oil Formation Volume Factor

Given

Density of Reservoir Oil		47.5 lbm/ft3

Density of Stock Tank Oil		55.5 lbm/ft3

Gas in Solution			400 Scf/STB

Gas Gravity				0.72

Definition

Bo = ρSTO+0.01357RsγgρoR

Calculation

Bo = 55.5+(0.01357)(400)(0.72)47.5=1.251 RBSTB

5.	Gas Formation Volume Factor

Given

Reservoir Pressure		1000 psi	Standard Pressure	14.65 psi

Reservoir Temperature             610 oR	Standard Temperature      60 oF

Gas Law Deviation Factor (z)  0.90

Definition

Bg = VrcVsc

Calculation

Bg = VrcVsc=nzRTprcnRTpsc=(0.9)(610)(14.65)(60+460)(1000)=0.0155RcfScf=0.00275RBScf

Fluid Properties Nomenclature

# Lower Case Letters

n	Amount, moles

nrc	Amount at reservoir conditions, moles

nsc	Amount at surface conditions, moles

p	Pressure, FL-2

prc	Pressure at reservoir conditions, FL-2 (psia)

psc	Pressure at surface conditions, FL-2 (psia)

z	Gas law deviation factor, actual volume/ideal volume

zrc	Gas law deviation factor at reservoir conditions, actual volume/ideal volume

zsc	Gas law deviation factor at surface conditions, actual volume/ideal volume (1.0)

# Upper Case Letters

B	Formation volume factor, reservoir volume/surface volume

Bg	Gas formation volume factor, reservoir volume/surface volume (rcf/scf)

Bo	Oil formation volume factor, reservoir volume/surface volume (rb/stb)

Bw	Water formation volume factor, reservoir volume/surface volume (rb/stb)

M	Molar mass, mass/mole

Ma	Apparent molar mass of a gas mixture, mass/mole (lbm/lb·mole)

Mair	Molar mass of air, mass/mole (28.97 lbm/lb·mole)

Mg	Molar mass of gas, mass/mole (lbm/lb·mole)

R	Gas-oil ratio, surface gas volume/surface oil volume

Rp	Produced gas-oil ratio, surface gas volume/surface oil volume (scf/stb)

Rs	Solution gas-oil ratio, surface gas volume/surface oil volume (scf/stb)

T	Temperature

V	Volume, L3

Vrc	Volume at reservoir conditions, L3 (ft3)

Vsc	Volume at surface conditions, L3 (ft3)

# Greek Letters

γl	Liquid specific gravity, density of liquid at 60oF/density of water at 60oF

ρ	Mass density, mass/volume

ρg	Density, mass/volume (lbm/ft3)

ρl	Liquid density, mass/volume (lbm/ft3)

ρo	Oil density, mass/volume (lbm/ft3)

ρoR	Oil density at reservoir conditions, mass/volume (lbm/ft3)

ρSTO	Oil density at surface (stock tank) conditions, mass/volume (lbm/ft3)

ρw	Water density, mass/volume (lbm/ft3)

Dimension and Unit Systems

Dimensions are physical quantities and units are standards of measurement.  Thus, dimensions are independent of units.

Dimensions are classified as fundamental, supplementary, and derived.  Supplementary dimensions can be considered as fundamental dimensions.  Fundamental dimensions are those necessary to describe a particular field of engineering or science.  Derived dimensions are combinations of fundamental dimensions.  A dimensional system is just the smallest number of fundamental dimensions to form a consistent and complete set for a field of engineering or science.  A dimensional system is called an absolute system if its dimensions are not affected by gravity; otherwise it is called a gravitational system.

Units are classified as base, supplementary, and derived.  Units for supplementary dimensions can also be considered as base units.  Derived units are combinations of base units.

A dimension and unit system is a set of fundamental dimensions and base units necessary for a particular field of science or engineering.  It is called a coherent system if equations between numerical values (units) have the same form as the corresponding equations between the quantities (dimensions).  For example, in the SI system, F = m·a is used to define the derived dimension.  Likewise, 1 newton = (1 kilogram)·(1 meter per second squared) is used to define the derived unit.  In other words, in a coherent system, combinations of any two unit quantities is the unit of the resulting quantity.  Coherency is a major advantage of the SI system.

In petroleum engineering, three dimension and unit systems are commonly used:

- The International System of Units (SI Units)

- The American Engineering System of Units (Oilfield Units)

- The Darcy System of Units (Darcy Units)

The purpose of these notes is to help you learn the above systems and how to convert units from one system to another.

The International System of Units (SI)

| Fundamental Dimension | Base Unit |
| --- | --- |
| length [L] | meter (m) |
| mass [M] | kilogram (kg) |
| time [t] | second (s) |
| electric current [I] | ampere (A) |
| absolute temperature [T] | kelvin (K) |
| luminous intensity [l] | candela (cd) |
| amount of substance [n] | mole (mol) |

| Supplementary Dimension | Base Unit |
| --- | --- |
| plane angle [] | radian (rad) |
| solid angle [] | steradian (sr) |

| Derived Dimension | Unit | Definition |
| --- | --- | --- |
| acceleration [L/t2] | meter per second squared | m/s2 |
| area [L2] | square meter | m2 |
| Celsius temperature [T ] | degree Celsius (oC) | K |
| concentration [n/L3] | mole per cubic meter | mol/m3 |
| density [M/L3] | kilogram per cubic meter | kg/m3 |
| electric charge [It] | coulomb (C) | A·s |
| electric potential [ML2/It3] | volt (V) | W/A |
| electric resistance [ML2/I2t3] | ohm () | V/A |
| energy [ML2/t2] | joule (J) | N·m |
| force [ML/t2] | newton (N) | kg·m/s2 |
| frequency [1/t] | hertz (Hz) | 1/s |
| molar mass [M/n ] | kilogram per mole | kg/mol |
| power [ML2/t3] | watt (W) | J/s |
| pressure [M/Lt2] | pascal (Pa) | N/m2 |
| quantity of heat [ML2/t2] | joule (J) | N·m |
| specific heat [L2/t2T] | joule per kilogram kelvin | J/(kg·K) |
| thermal conductivity [ML/t3T] | watt per meter kelvin | W/(m·K) |
| velocity [L/t] | meter per second | m/s |
| viscosity, dynamic [M/Lt] | pascal second | Pa·s |
| volume [L3] | cubic meter | m3 |
| work [ML2/t2] | joule (J) | N·m |

The International System of Units (SI) (Cont'd)

| Prefix | Decimal Multiplier | Symbol |
| --- | --- | --- |
| atto | 10-18 | a |
| femto | 10-15 | f |
| pico | 10-12 | p |
| nano | 10-9 | n |
| micro | 10-6 |  |
| milli | 10-3 | m |
| centi | 10-2 | c |
| deci | 10-1 | d |
| deka | 10+1 | da |
| hecto | 10+2 | h |
| kilo | 10+3 | k |
| mega | 10+6 | M |
| giga | 10+9 | G |
| tera | 10+12 | T |
| peta | 10+15 | P |
| exa | 10+18 | E |

American Engineering System of Units (AES)

| Fundamental Dimension | Base Unit |
| --- | --- |
| length [L] | foot (ft) |
| mass [M] | pound mass (lbm) |
| force [F] | pound force (lbf) |
| time [t] | second (sec) |
| electric charge[Q] | coulomb (C) |
| absolute temperature [T] | Rankine (oR) |
| luminous intensity [l] | candela (cd) |
| amount of substance [n] | mole (mol) |

| Supplementary Dimension | Base Unit |
| --- | --- |
| plane angle [] | radian (rad) |
| solid angle [] | steradian (sr) |

| Derived Dimension | Unit | Definition |
| --- | --- | --- |
| acceleration [L/t2] | foot per second squared | ft/sec2 |
| area [L2] | square foot | ft2 |
| Fahrenheit temperature [T ] | degree Fahrenheit (oF) | oR-459.67 |
| concentration [n/L3] | mole per cubic foot | mol/ft3 |
| density [M/L3] | pound mass per cubic foot | lbm/ft3 |
| electric current [Q/t] | ampere (A) | C/sec |
| electric potential [FL/Q] | volt (V) | W/A |
| electric resistance [FLt/Q2] | ohm () | V/A |
| energy [FL] | foot pound force | ft· lbf |
| frequency [1/t] | hertz (Hz) | 1/sec |
| molar mass [M/n ] | pound mass per mole | lbm/mol |
| power [FL/t] | foot pound force per second | ft· lbf/sec |
| pressure [F/L2] | pound force per square foot (psf) | lbf/ft2 |
| quantity of heat [FL] | british thermal unit (BTU) | 777.65 ft· lbf |
| velocity [L/t] | foot per second | ft/sec |
| viscosity, dynamic [Ft/L2] | pound force second per square foot | lbf·sec/ft2 |
| volume [L3] | cubic foot | ft3 |
| work [FL] | foot pound force | ft· lbf |

Oilfield Units (related to AES System)

| Dimension | Unit | Definition |
| --- | --- | --- |
| area [L2] | acre (ac) | 43,560 ft2 |
| energy [FL] | horsepower hour (hp·hr) kilowatt hour (kW·hr) | 1.98000 x 106 ft· lbf 2.6552 x 106 ft· lbf |
| length [L] | inch (in) yard (yd) mile (mi) | 1/12 ft 3 ft 5,280 ft |
| mass [M] | ounce (oz) ton | 1/16 lbm 2000 lbm |
| power [FL/t] | horsepower (hp) watt (W) | 550 ft· lbf/sec 0.73756 ft· lbf/sec |
| pressure [F/L2] | pound force per square inch (psi) atmosphere (atm) | 144 lbf/ft2 14.696 psi |
| time [t] | minute (min) hour (hr) day | 60 sec 3,600 sec 86,400 sec |
| viscosity, dynamic [Ft/L2] | centipoise (cp) | 10-2 dyne·s/cm2 |
| volume [L3] | gallon (gal) barrel (bbl) acre·ft (ac·ft) | 0.133681 ft3 5.614583 ft3 43,560 ft3 |

Conversion of Units

In a coherent system of units such as SI, a derived dimension is a product or quotient of other dimensions.  For example, the dimension—force—is the product of mass and acceleration,         F = ma, and the unit of force—newton—is the product of the unit of mass and the unit of acceleration.  In the American Engineering System, force is expressed in lbf, mass in lbm, and acceleration in ft/sec2.  This system is obviously not coherent.  Hence, a conversion factor other than one must be used in the equation; that is, F = magc, where gc = 32.174lbm⋅ftsec2⋅lbfis a constant, known as the gravitational conversion constant.

Derivation of gc

The principle of conservation of units is used to derive gc.  Briefly stated, this principle is that to convert a relationship (an equation) from a given system of units to another (a required) system of units, the given units must be conserved.  The technique is illustrated below.

We wish to convert F = ma from SI to AES.  Here, SI is the given system and AES is the required system.

First, find the dimension of the “hidden constant” in the given equation.  In other words,

1 = Fma has dimension force divided by the product of mass and acceleration.

Next, consider the units of the hidden constant—the given units are

N⋅s2kg⋅m and the required units arelbf⋅sec2lbm⋅ft.

Now, convert the given units of the hidden constant to the required units.

Thus,

1 N⋅s2kg⋅m0.45359 kglbm0.3048 mftlbf4.4482 N= 132.174lbf⋅sec2lbm⋅ft

Review Problems

| 1. A weir is a regular obstruction in an open flow channel over which flow takes place such as that shown in the sketch at right. It can be used to measure open channel flow rates. For a rectangular weir, the theoretical formula for the flow rate is Q = 5.35LH1.5 where Q is discharge rate in ft3/sec, L is length of the weir in ft, H is height of fluid above the crest in ft. Determine a new constant so the formula can be applied with Q in m3/s and L and H in m. |  |
| --- | --- |

2.  The ideal gas equation can be written,

pv = RT

where   p = pressure, Pa

v = molar volume, m3/(kg·mol)

R = gas constant, 8314.5 Pa·m3/(kg·mol·K)

T = absolute temperature, K

Determine a new constant so that the equation can be applied with p in lbf/in2, v in ft3/(lbm·mol), and T in oR.

3. The universal law of gravity may be written,

F=6.672 x 10-11m1m2r2

where   F = force of attraction between two bodies, N

m1 = mass of body one, kg

m2 = mass of body two, kg

r = distance between bodies, m

Determine a new constant so that the law can be applied with F in lbf, m1 & m2 in lbm, and r in mi.

#### Answers

1.   2.95 m0.5/s

2.  10.732 psi· ft3/(lbm·mol· oR)

3.   1.192 x 10-18 lbf·mi2/lbm2

#### Porosity, Permeability, and Saturation (-k-S)

Porosity is a measure of the fluid storage capacity of a rock,

φ=VpVb

where     = porosity, fraction

Vb = bulk volume = Vp + Vm

Vp = pore volume

Vm = matrix volume

| Permeability is a measure of the fluid conductivity of a rock. It is defined by Darcy’s law, which is based upon ex-perimental data. For horizontal, linear flow of a liquid completely saturating the rock, k=qμLAΔp where k = permeability, d q = flow rate, cm3/s  = fluid viscosity, cp L = length of flow path, cm A = cross-sectional area of flow path, cm2 p = pressure difference across flow path, atm The dimension of permeability is [L2]. qμLAΔp=L3t⋅P⋅t1⋅L1⋅1L2⋅1P |  |
| --- | --- |

Saturation is a measure the amount and type of fluid stored in a rock.

Sℓ=VℓVp

where   Sℓ= saturation of fluid ℓ (oil, water, or gas)

Vℓ = fluid volume (ℓ= oil, water, or gas)

Vp = pore volume

Darcy’s Law

| Darcy found that when water flows vertically downward through sand, the volume of water passing through the system in unit time (the discharge q in Fig. 2) is proportional to the drop in head h across the sand. Considering the cross sectional area A and thick-ness of the sand l, these observations can be written, v=qA=−Kdhdl where K is a coefficient depending upon the permeability of the sand and the properties of the fluid. Darcy’s original experiment has been extended to answer questions about flow of different fluids in porous media of various permeabilities along flow paths in various directions. |  |
| --- | --- |

The generalized two dimensional form of Darcy’s law (API Code 27) is

vs=−kμdpds−ρg1.01325dzdsx 10−6

where	 s = distance in direction of flow (always positive), cm

vs = volume flux across a unit area of the porous medium in unit time (q/A)

along flow path s, cm/s

z = vertical coordinate (positive downward), cm

 = density of the fluid, g/cm3

g = acceleration of gravity, 980.665 cm/s2

dp/ds = pressure gradient along path s at the point to which vs refers, atm/cm

 = viscosity of the fluid, cp

k = permeability of the porous medium, d

The Darcy System of Units

| Fundamental Dimension | Base Unit |
| --- | --- |
| length [L] | centimeter (cm) |
| mass [M] | gram (g) |
| time [t] | second (s) |

| Derived Dimension | Unit | Definition |
| --- | --- | --- |
| acceleration [L/t2] | centimeter per second squared | cm/s2 |
| area [L2] | square centimeter | cm2 |
| density [M/L3] | gram per cubic centimeter | g/cm3 |
| energy [ML2/t2] | erg | dyne·cm |
| force [ML/t2] | dyne | g·cm/s2 |
| permeability [L2] | darcy (d) | cp·cm2/atm·s |
| pressure [M/Lt2] | atmosphere (atm) | 1.013250 x 106 dyne/cm2 |
| velocity [L/t] | centimeter per second | cm/s |
| viscosity, dynamic [M/Lt] | centipoise (cp) | 10-2 dyne·s/cm2 |
| volume [L3] | cubic centimeter | cm3 |
| work [ML2/t2] | erg | dyne·cm |

# In SI Units

| Derived Dimension | Unit |
| --- | --- |
| permeability [L2] | square meter (m2) |

# In Oilfield Units

| Derived Dimension | Unit |
| --- | --- |
| permeability [L2] | millidarcy (md) |

Converting Permeability Units

The principle of conservation of units is used.  Briefly stated, this principle is that to convert a relationship (an equation) from a given system of units to another (a required) system of units, the given units must be conserved.  The technique is illustrated below.

We wish to convert k=qμLAΔp from Darcy units (d, cm, s, cp, atm) to Oilfield units (md, ft, bbl, day, cp, psi).  Here, Darcy units is the given system and Oilfield units is the required system.

First, find the dimension of the “hidden constant” in the given equation.  In other words,

1 = kAΔpqμL has dimension kAp divided by qL [1].

Next, consider the units of the hidden constant—the given units are

d⋅atm⋅scp⋅cm2 and the required units aremd⋅ft⋅psi⋅daycp⋅bbl.

Now, convert the given units of the hidden constant to the required units.

Thus,

1 d⋅atm⋅scp⋅cm2103mddftft14.696 psiatmday86,400 s30.482cm2ft2=

158.02 md⋅ft⋅psi⋅daycp⋅ft35.614583 ft3bbl=887.22 md⋅ft⋅psi⋅daycp⋅bbl

Converting from Darcy units to other systems of units is similar.

Summary of Darcy equations for horizontal, linear flow (including conversion factors):

| ### Darcy Units | ### SI Units | ### Oilfield Units |
| --- | --- | --- |
| k=qμLAΔp | k=qμLAΔp | k=887.2qμLAΔp |

NOTE:  A m2 is a huge permeability unit!  There are 1.01325 x 1012 d/m2.

Thus, the SPE preferred SI unit for permeability is a m2, introducing

a factor of 1012 in the equation.

Horizontal, Linear Flow

A

q 								q

L

x1			       x2

Applying Darcy’s law,

vx=qA=−kμdpdx

Separate the variables and integrate to obtain the flow equation,

qx1x2dx=−kAμp1p2dp ⇒ q(cm3s)=−kAp2−p1μx2−x1=kAp1−p2μx2−x1=kAΔpμL

In oilfield units, the equation is

q(RBday)=1.1271 x 10−3kAΔpμL or q(STBday)=1.1271 x 10−3kAΔpμBL

EXAMPLE:  A rock sample10 cm long and 2 cm2 in cross section is used for some steady-state

flow tests.  Calculate the permeability of the rock if it is completely saturated with

an oil having a viscosity of 2.5 cp and oil is flowed through the rock at a rate of

0.0080 cm3/s under a 1.5 atm pressure drop?

k=qμLAΔp=0.0080 cm3s2.5 cp10 cm2 cm21.5 atm=0.067 d=67 md

Wellbore Volumes

Wellbores are right circular cylinders,

the volume of which is,

V=πr2h=π4d2h			     		      h

Many drilling and completion applications

require the annular volume,

V=πr22−r12h=π4d22−d12h

EXAMPLE:   Suppose you are the drilling engineer on a rig.  You have set 7-5/8 inch OD,

33.7 lbm/ft casing (ID = 6.765 inches) from 0 to 5900 ft and have drilled out to

7900 ft. The average bore hole diameter from 5900 to 7900 ft is 6.25 inches.

Calculate the annular volume in ft3 if 4-1/2 inch OD casing is installed.

V=π46.765 in12 inft2−4.5 in12 inft25900 ft+6.25 in12 inft2−4.5 in12 inft22000 ft

V =  987 ft3

Pressure Gradients

The basic equation of hydrostatics can be written

p = gh

where p = change in fluid pressure, Pa

 = density, kg/m3

g  = acceleration of gravity, m/s2

h  = change in height of fluid, m

ΔpΔh=ρgThe pressure gradient (Pa/m) is

There is no conversion factor in SI because

Pam=kgm3⋅ms2=Nm2⋅1m=Pam

In oilfield units, p = change in fluid pressure, lbf/in2 or psi

 = density, lbm/ft3

g = acceleration of gravity, ft/s2

h = change in height of fluid, ft

and the pressure gradient (psi/ft) is

ΔpΔh=ρg144gc, where  gc=32.174lbm⋅ftlbf⋅s2

Notice that the units check because

psift=lbmft3⋅fts2⋅ft2in2⋅lbf⋅s2lbf⋅ft=lbfin2⋅1ft=psift

EXAMPLE:  Calculate the pressure gradient for a fluid having a density of 65 lbm/ft3.

Assume g = 32 ft/s2.

ΔpΔh=ρg144gc=653214432.174=0.449 or 0.45 psift

Horizontal, Radial Flow

ds = -dr

r

h

rw			                      re

Applying Darcy’s law,

vr=qA=q2πrh=kμdpdr

Separate the variables and integrate to obtain the flow equation,

qrwredrr=2πkhμpwpedp ⇒ q(cm3s)=2πkhpe−pwμlnre/rw=2πkhΔpμlnre/rw

In oilfield units, the equation for liquid is

qℓ(RBday)=7.0819 x 10−3khΔpμlnre/rw or qℓ(STBday)=7.0819 x 10−3khΔpμBℓlnre/rw

and, the equation for gas is

qg(RBday)=7.0819 x 10−3khΔpμlnre/rw or qg(Scfday)=7.0819 x 10−3khΔpμBglnre/rw

Reservoir Volumes

h

rw			                      re

The volume occupied by the reservoir rock or bulk volume is

Vb=πre2h

Since porosity is the ratio of the pore volume to bulk volume, the pore volume is

Vp=πre2hφ

And, since water saturation is the fraction of the pore volume filled with water, the hydrocarbon pore volume is

Vhc=πre2hφ1−Sw

A commonly used equation to calculate the surface volume of oil in place is

N(STB)=7758Ahφ1−SwBo,where A is the drainage area in acres.

and the surface volume of gas in place is

G(Scf)=7758Ahφ1−SwBg,where Bg is the gas formation volume factor in RBScf.

Homework Problems

1.  A certain oilfield has an average producing gas-oil ratio of 350 scf/stb.  The density of the stock tank oil is 54.7 lbm/ft3 at 60 oF and the apparent molar mass of the associated gas is 21.7 lbm/lbmole.  Determine the

a.  API gravity of the stock tank oil.

b.  Specific gravity of the associated gas.

c.  Density of the reservoir oil if the oil formation volume factor is 1.2 rb/stb and the gas in

solution is equal to the average producing gas-oil ratio.

d.  Type of reservoir fluid.

2.  The equation of state for a particular substance at low pressures can be written

p = 53.55T,

where	 p = pressure, lbf/ft2

= density, lbm/ft3

T = absolute temperature, oR

a.  Determine a new constant so the equation can be used with p in Pa,  in kg/m3, and T in K.

b.  Use the ideal gas law (pV = nRT), the definition of quantity (mass/molar mass), and the

definition of density (mass/volume) to identify the substance.

3.  Power is defined as work per unit time.

a.  Find the units of the hidden constant in P = W/t (SI system), where P is power in

watts, W is work in newton-meters, and t is in seconds.

b.  Convert to constant to the AES system, where P is power in horsepower, W is work in

foot-pounds of force, and t is in minutes.

4.  A cylindrical sandstone sample, one inch in diameter and one inch long has a pore volume of 0.275 in3.  The pores are partially filled with 0.135 in3 of water.  What is the porosity and water saturation of the sample?

5.  A cylindrical sandstone sample, one inch in diameter and one inch long is placed in a permeameter and water having a viscosity of one centipoise is flowed through the circular faces of the sample at a rate of one cm3/s under a pressure difference of one atmosphere.  What is the permeability of the sample?

6.  An acre-ft of reservoir has a bulk volume of 43,560 ft3.  Calculate the volume of water (in barrels per acre-ft) for a sandstone reservoir having a porosity of 20 % and a water saturation of 25 %.

7.  A rock sample is 1 inch long and 1 inch in diameter.  The sample is used for some steady-state flow tests.  Calculate the permeability of the rock if it is completely saturated with water having a viscosity of 1 cp and water is flowed through the sample at a rate of 0.5 in3/min and the pressure drop is 10 psi.

8.  Find the conversion constant to calculate permeability in md from a horizontal, linear flow experiment when flow rate is in cm3/s, viscosity is in cp, length is in cm, and pressure drop is in dynes/cm2.

9.  Find the conversion constant to calculate permeability in md from a horizontal, linear flow experiment when flow rate is in m3/day, viscosity is in cp, length is in m, and pressure drop is in kPa.

10.  Suppose you are the drilling engineer on a rig.  You have set 9 5/8 in OD, 40 lbm/ft casing  (ID = 8.835 in) from 0 to 2000 ft and have drilled a 7 7/8 in bore hole from 2000 to 4000 ft.

a.  Find the capacity of the hole in barrels from 0 to 4000 feet.

b.  The hydrostatic pressure at 4000 ft with fresh water in hole.

c.  The annular volume if 4.5 inch OD, 11.60 lbm/ft casing is installed from 0 to 4000 ft

with fresh water in hole.

11.   Find the conversion constant to calculate pressure gradient in psi/ft when fluid density is in lbm/gal (ppg)

12.  A vertical gas well is 6000 ft deep.

a.  What is its bottom hole temperature if the mean surface temperature is 60 oF and the

local temperature gradient is 1.1 oF/100 ft?

b.  What is its bottom hole flowing pressure if the surface flowing pressure is 900 psi

and the pressure gradient in the well bore is 0.15 psi/ft?

13.  Assume a vertical well drains a right cylindrically shaped oil reservoir for which you have the following data:

drainage area				40 acres (external radius = 745 ft)

formation thickness			25 feet

radius of well bore		    	0.25 feet

porosity				19 %

water saturation				21 %

permeability			     	75 millidarcy

oil viscosity			     	2.1 centipoise

pressure at external radius	    	3545 psi

pressure at the well bore radius		1515 psi

Calculate the initial production rate in reservoir barrels per day.

14.  Given the following data on a gas reservoir:

porosity				0.22

water saturation				0.30

gas formation volume factor		1.0 rb/MScf

a.  Calculate the volume of gas in MScf per acre-foot of reservoir volume.

b.  If the recovery factor is 80 %, what is the recoverable reserve of a 160 acre spaced well

with an average reservoir thickness of 15 feet?

# Problem Layout

Conversion Factors

| To convert from | ##### Multiply by | To obtain |
| --- | --- | --- |
| acres | 4.356 x 104 | sq feet |
| acres | 4.047 x 103 | sq meters |
| acre-feet | 4.356 x 104 | cubic feet |
| atmospheres | 7.6 x 101 | cm of mercury at 0 oC |
| atmospheres | 3.39 x 101 | ft of water at 4 oC |
| atmospheres | 1.01325 x 105 | pascals |
| atmospheres | 1.4696 x 101 | pounds-force/sq in |
| barrels(oil) | 5.614583 | cubic feet |
| barrels(oil) | 4.2 x 101 | gallons |
| barrels(oil)/d | 2.295 x 10-3 | cubic meters/s |
| Btu | 1.0550 x 1010 | ergs |
| Btu | 7.7816 x 102 | ft-pounds |
| Btu | 3.927 x 10-4 | horsepower-hours |
| Btu | 1.055 x 103 | joules |
| Btu | 2.928 x 10-4 | kilowatt-hours |
| centimeters | 3.281 x 10-2 | feet |
| centimeters | 3.937 x 10-1 | inches |
| centimeters | 6.214 x 10-6 | miles |
| cm of mercury | 1.316 x 10-2 | atmospheres |
| cm of mercury | 4.461 x 10-1 | ft of water |
| cm of mercury | 1.333 x 103 | pascals |
| cm of mercury | 1.934 x 10-1 | pounds-force/sq in |
| cm/s | 1.969 | ft/min |
| cm/s | 3.281 x 10-1 | ft/s |
| cm/s | 3.6 x 10-2 | kilometers/h |
| cm/s | 2.237 x 10-2 | miles/h |
| centipoise | 1.0 x 10-3 | pascal-seconds |
| cubic cm | 6.28982 x 10-6 | barrels(oil) |
| cubic cm | 3.531 x 10-5 | cubic ft |
| cubic cm | 2.642 x 10-4 | gallons |
| cubic cm | 1.0 x 10-3 | liters |
| cubic cm/s | 5.434 x 10-1 | barrels(oil)/d |
| cubic feet | 1.781 x 10-1 | barrels(oil) |
| cubic feet | 2.832 x 10-2 | cubic meters |
| cubic feet | 7.48052 | gallons |
| cubic feet | 2.832 x 101 | liters |
| cubic ft/d | 3.278 x 10-7 | cubic meters/s |
| cubic feet/h | 4.275 | barrels(oil)/d |
| cubic feet/min | 4.720 x 10-1 | liters/s |
| cubic feet/s | 4.48831 x 102 | gallons/min |
| cubic meters | 6.28982 | barrels(oil) |
| cubic meters | 3.531 x 101 | cubic ft |
| To convert from | ##### Multiply by | To obtain |
| cubic meters | 2.642 x 102 | gallons |
| cubic meters | 1.0 x 103 | liters |
| cubic meters/s | 5.434 x 105 | barrels(oil)/d |
| darcies | 9.869233 x10-13 | sq meters |
| dynes/sq cm | 9.869233 x 10-7 | atmospheres |
| dynes | 1.0 x 10-5 | joules/meter (newtons) |
| dynes | 2.248 x 10-6 | pounds-force |
| ergs | 9.486 x 10-11 | Btu |
| ergs | 7.376 x10-8 | ft-pounds |
| ergs | 1.0 x 10-7 | joules |
| ergs | 2.773 x 10-14 | kilowatt-hrs |
| ergs/s | 1.341 x 10-10 | horsepower |
| feet | 3.048 x 10-4 | kilometers |
| feet | 3.048 x 10-1 | meters |
| feet | 1.894 x 10-4 | miles |
| feet of water | 2.950 x 10-2 | atmospheres |
| feet of water | 2.242 | cm of mercury |
| feet of water | 2.989 x 103 | pascals |
| feet of water | 4.335 x 10-1 | pounds-force/sq in |
| feet/min | 1.829 x 10-2 | km/hr |
| feet/min | 3.048 x 10-1 | meters/min |
| feet/min | 1.136 x 10-2 | miles/h |
| feet/s | 1.097 | km/h |
| feet/s | 1.829 x 101 | meters/min |
| feet/s | 6.818 x 10-1 | miles/h |
| feet/s2 | 3.048 x 10-1 | meters/s2 |
| foot-pounds | 1.286 x 10-3 | Btu |
| foot-pounds | 1.356 x 107 | ergs |
| foot-pounds | 1.356 | joules |
| foot-pounds | 1.383 x 10-1 | kg-m |
| foot-pounds | 3.766 x 10-7 | kilowatt-h |
| foot-pounds/min | 1.286 x 10-3 | Btu/min |
| foot-pounds/min | 3.030 x 10-5 | horsepower |
| foot-pounds/min | 2.260 x 10-5 | kilowatts |
| foot-pounds/s | 4.6263 | Btu/h |
| foot-pounds/s | 1.818 x 10-3 | horsepower |
| foot-pounds/s | 1.356 x 10-3 | kilowatts |
| gallons | 2.381 x 10-2 | barrels(oil) |
| gallons | 1.337 x 10-1 | cubic ft |
| gallons | 3.785 x 10-3 | cubic meters |
| gallons | 3.785 | liters |
| gallons/min | 2.228 x 10-3 | cu ft/s |
| grams | 2.205 x 10-3 | pounds-mass |
| horsepower | 4.244 x 101 | btu/min |

| To convert from | ##### Multiply by | To obtain |
| --- | --- | --- |
| horsepower | 3.3 x 104 | ft-lb/min |
| horsepower | 5.50 x 102 | ft-lb/s |
| horsepower | 7.457 x 10-1 | kilowatts |
| horsepower-hours | 2.547 x 103 | Btu |
| horsepower-hours | 1.98 x 106 | ft-lb |
| horsepower-hours | 2.684 x 106 | joules |
| inches | 2.540 | centimeters |
| inches | 2.540 x 10-2 | meters |
| joules | 9.486 x 10-4 | Btu |
| joules | 7.376 x 10-1 | ft-pounds |
| joules/s | 5.6907 x 10-2 | Btu/min |
| kilograms | 2.2046 | pounds-mass |
| kilograms/cu m | 6.243 x 10-2 | pounds-mass/cu ft |
| kilometers | 3.281 x 103 | ft |
| kilometers | 6.214 x 10-1 | miles |
| kilometers/h | 5.468 x 101 | ft/min |
| kilometers/h | 9.113 x 10-1 | ft/s |
| kilometers/h | 6.214 x 10-1 | miles/h |
| kilowatts | 1.341 | horsepower |
| kilowatt hour | 3.413 x 103 | Btu |
| kilowatt hour | 2.655 x 106 | ft-lb |
| kilowatt hour | 3.6 x 106 | joules |
| liters | 1.0 x 103 | cubic cm |
| liters | 3.531 x 10-2 | cubic ft |
| liters | 1.0 x 10-3 | cubic meters |
| liters | 2.642 x 10-1 | gallons |
| liters/s | 1.5850 x 101 | gallons/min |
| meters | 3.281 | ft |
| meters | 3.937 x 101 | inches |
| meters | 6.214 x 10-4 | miles |
| meters | 1.094 | yards |
| meters/s | 1.968 x 102 | ft/min |
| meters/s | 3.281 | ft/s |
| meters/s | 3.6 | km/h |
| meters/s | 2.237 | miles/h |
| miles | 5.280 x 103 | ft |
| miles | 1.609 | kilometers |
| miles | 1.609 x 103 | meters |
| miles/h | 8.8 x 101 | ft/min. |
| miles/h | 1.467 | ft/s |
| miles/h | 1.6093 | km/h |
| miles/h | 2.682 x 101 | meters/min |
| newtons | 1.0 x 105 | dynes |
| pounds-force | 4.4482 x 105 | dynes |

| To convert from | ##### Multiply by | To obtain |
| --- | --- | --- |
| pounds-force | 4.4482 | newtons |
| pounds-mass | 4.5359 x 102 | grams |
| pounds-mass | 4.5359 x 10-1 | kilograms |
| pounds-mass/cu ft | 1.602 x 101 | kg/cu m |
| pounds-force/sq ft | 4.725 x 10-4 | atmospheres |
| pounds-force/sq ft | 1.602 x 10-2 | ft of water |
| pounds-force/sq ft | 4.788 x 10 | pascals |
| pounds-force/sq in | 2.307 | ft of water |
| pounds-force/sq in | 5.171 | cm of mercury |
| pounds-force/sq in | 6.895 x 103 | pascals |
| radians | 5.7296 x 101 | degrees |
| radians/s | 9.5493 | revolutions/min |
| revolutions/min | 1.047 x 10-1 | radians/s |
| square cm | 1.076 x 10-3 | sq ft |
| square feet | 2.296 x 10-5 | acres |
| square feet | 9.29 x 10-2 | sq m |
| square feet | 3.587 x 10-8 | sq miles |
| square meters | 2.471 x 10-4 | acres |
| square meters | 1.076 x 101 | sq ft |
| square meters | 3.861 x 10-7 | sq miles |
| square miles | 2.788 x 107 | sq ft |
| square miles | 2.590 x 106 | sq meters |
| watts | 3.4129 | Btu/h |
| watts | 1.0 x 107 | ergs/s |
| watts | 4.427 x 101 | ft-lb/min |
| watts | 7.376 x 10-1 | ft-lb/s |
| watts | 1.341 x 10-3 | horsepower |
| watt hours | 3.413 | Btu |
| watt hours | 2.656 x 103 | ft-pounds |